Intelligent Patient Monitor

Next-generation EMS monitoring — on-device AI, automatic documentation, and structured handoff · project brief

Concept. An intelligent patient monitor that replaces the one EMS crews already carry. Providers attach ECG leads, a blood-pressure cuff, and a pulse-oximeter exactly as they do today — the monitor fuses those signals, runs AI on the device (no cloud), automatically documents the encounter as a timestamped clinical timeline, and hands the receiving hospital a structured summary. Core monitoring (ECG, blood pressure, pulse oximetry, temperature) with optional modules (digital stethoscope, AI-guided ultrasound, thermal). It preserves the existing EMS workflow while making monitoring and handoff substantially more informative.

Status. A pre-seed concept with a working software proof. Three real clinical signal-interpretation models learn and deploy on the committed on-device AI chip (BrainChip Akida) with almost no on-chip accuracy loss — ECG arrhythmia (MIT-BIH), heart-sound analysis (CinC 2016), and ECG myocardial-infarction detection (PTB-XL); an occult-hemorrhage (compensatory-reserve) model is demonstrated in simulation. Now at hardware bring-up: Akida on BrainChip's cloud FPGA and a sensor-integrated prototype on an off-the-shelf Akida board, with sensor acquisition and the fusion/timeline software in parallel. No custom silicon or PCB is on the critical path.

1. The real problem — it isn't sensors

Sensors are not the gap. Nearly every modality EMS needs already exists as a portable device — ECG, pulse oximetry, blood pressure, temperature, digital stethoscope, capnography, ultrasound, thermal. The gap is what happens with the data: documentation is manual, patient handoff is fragmented, information is scattered across devices, and the clinical context — what changed, when, and how the patient responded to treatment — is lost or reconstructed from memory. Monitors record numbers; none capture the story.

2. What the product does

It replaces the existing monitor with no change to workflow — leads, cuff, and pulse-ox go on exactly as before. The difference is what the monitor does with the signals: it fuses them on-device, detects clinically meaningful events and trends, documents automatically, and prepares the hospital handoff.

The differentiator — an automatic clinical timeline. Instead of storing every measurement, the monitor records meaningful, trend-aware clinical events — far more useful to the receiving hospital than a table of vitals:

08:13  Baseline vitals established

08:19  BP decreased 23%  ->  hypotension (94/58)

08:20  SpO2 desaturation (91%)

08:21  IV fluids started; HR +18 bpm

08:25  BP improving after treatment

08:33  Vitals stable for 10 min

Passive AI, clinician in control. The AI does not declare diagnoses. It surfaces observations and trends — blood pressure decreasing, perfusion worsening, shock index rising — with a confidence score and a recommendation to reassess or repeat a measurement. It never states “the patient has hemorrhagic shock.” Clinicians keep the judgment; the monitor keeps the record.

3. Hardware — a core monitor plus modules

Not an “everything device.” A core monitor covers what goes on nearly every patient; specialized capabilities are optional modules. This keeps cost, complexity, and regulatory burden down, and lets crews carry only what they need.

| Tier | Capability | Role |
| --- | --- | --- |
| Core monitor | ECG · blood pressure · pulse oximetry (PPG) · temperature | on nearly every patient — the platform |
| Optional module | Digital stethoscope (heart/lung sounds) | added when auscultation is needed |
| Optional module | Portable ultrasound — AI-GUIDED, not AI-diagnosed | most-requested add-on; assists image capture |
| Optional module | Thermal imaging | value is when FUSED with PPG/BP/ECG for perfusion/shock, not standalone |

Thermal is optional because it reads skin (not core) temperature, is hard to interpret, and few EMS protocols use it today; its real value is fused perfusion/shock estimation. Blood analyzers and EEG are niche, optional at most.

4. Why on-device (neuromorphic) AI

Continuous multimodal fusion, event and trend detection, and documentation all run on the device — offline, low power, no cloud. That matters in transport (no reliable network), for patient privacy, and for latency. A neuromorphic core (BrainChip Akida) is what makes always-on multimodal AI feasible inside a monitor's power and thermal budget. Models are trained off-device, quantized to the chip's integer arithmetic, and verified on the Akida simulator before any hardware is bought — a chip-agnostic pipeline already validated end to end.

5. What's proven

The on-device interpretation models that make the monitor smart already learn on real clinical data and deploy on the committed chip with almost no accuracy loss (5-seed measurements, public datasets):

ECG arrhythmia (MIT-BIH): AUROC 0.970, float→on-chip agreement 0.98.

Heart sounds (CinC 2016): AUROC 0.945, agreement 0.95.

ECG myocardial infarction (PTB-XL): AUROC 0.890, agreement 0.92.

Occult hemorrhage / compensatory reserve: demonstrated in physiologically-grounded simulation (real induced-hypovolemia data requested; not yet a clinical accuracy claim).

Each result is leakage-safe, multi-seed, and reported with per-class recall. On-chip numbers are from BrainChip's Akida simulator (not a published bit-exact-to-silicon claim).

6. The differentiator (moat)

Not a new sensor — every modality is a commodity handheld already. The moat is integration plus the clinical record: on-device multimodal fusion feeding an automatic, trend-aware clinical timeline and a structured hospital handoff, inside a monitor that preserves EMS workflow. Competitors own single modalities; none own the unified, self-documenting record.

7. Why it doesn't exist yet

Regulation: every added sensor adds clearance burden — the core-plus-modules design contains it.

Engineering: combining ECG, ultrasound, PPG, thermal, and audio into one reliable battery device is hard.

Market structure: different companies dominate different modalities; none integrate.

Workflow: EMS crews won't adopt another device — so we replace the monitor they already use, not add one.

8. Market & go-to-market

Primary: civilian EMS / ambulance services — the monitor is a device every crew already carries and must eventually replace, so adoption needs no new workflow. Adjacent: far-forward, military, and disaster medicine, where the offline, low-power, rugged story is strongest and DARPA (MASH, Triage Challenge) and TATRC offer a non-dilutive on-ramp.

9. Regulatory

Position as passive decision-support with the clinician in control — observations and trends, not diagnoses — which keeps it out of the highest-risk lane. The core monitor and each module carry their own clearances, and 97% of AI devices clear via a 510(k) predicate. Modular design contains the regulatory burden per capability.

10. Where it stands & the ask

Software and Akida-simulator validation are done across three real modalities; the project is at hardware bring-up on an off-the-shelf Akida board, with the sensor-acquisition and fusion/timeline software in parallel. Next: the multimodal fusion + clinical-timeline layer, a measured on-device energy figure, and the core-monitor prototype. The raise funds hardware (Akida dev hardware, sensors), clinical-data access, and the engineering to carry a validated pipeline into a field-ready core monitor.

Sources

Preventable prehospital death & MARCH  ·  POC testing in austere care  ·  digital-stethoscope & PPG CRM validation  ·  DARPA MASH  ·  BrainChip Akida  ·  FDA AI/SaMD pathway